Location: Nantes

ATMEL CONFIDENTIAL

| Document title ATxmega128B1 USB DFU Bootloader Configuration for production |  |  |  | Page 1 of 2 |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Product/project AVR ATxmega128B1 |  |  |  |  | Document no. |  |  |  |
| Department: IC Tools Apps Tech QA ADM IT HMS Access Open Internal Confidential Status 0 Not created 1 Not approved 2 Approved with remarks 3 Approved |  |  |  |  |  |  |  |  |
| 0 | 2012-02-20 | Initial revision. | RL |  |  |  |  | 1 |
| Rev. | Date | Description | Prep. |  |  | Check. | Appr. | Status |

# Scope

This document describes the non-volatile settings (General-purpose fuses) of the ATxmega128B1 USB DFU Bootloader that should be loaded on ATxmega128B1 USB bootloader samples.

# References

- ATxmega128B1 datasheet

- AVR1916 application note

# General-Purpose Fuse Settings

The fuses are configured as mentioned in the ATxmega128B1 datasheet, excepted:

- Boot Loader Reset

- BOD operation in power-down mode

- enable of lock Bits of programming interface.

Also, the datasheet does not mention the default value for fuses 4 and 5, and we choose:

- to enable BOD with very low level

- to erase EEPROM during chip erase

Thus, the fuses values are:

Fuse 0 = 0x00

Fuse 1 = 0x00

Fuse 2 = 0xBE

Fuse 4 = 0xFE

Fuse 5 = 0xEF

Lock byte = 0xFC

# Bootloader firmware

The atxmega128b1_bootloader_usb_dfu_103.hex should be loaded together with the previous fuses setting configuration into the part..